SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

Real-Time Embedded System I/O Communication Mechanism

Problem

Construct an I/O communication mechanism for a real-time embedded system with:

- CPU utilization below 15%

- Multiple sensors generating interrupts every 1 ms

- Vectored interrupt handling

- Selection between memory-mapped I/O and I/O-mapped I/O for optimal latency.

Proposed Architecture

Sensors → Interrupt Controller → CPU

CPU ↔ Memory Bus ↔ Sensor Registers

The system uses memory-mapped I/O because device registers can be accessed using normal load/store instructions, providing simple and fast register access.

Working

- Multiple sensors continuously monitor physical parameters.

- Every 1 ms, sensors generate interrupt requests.

- The interrupt controller receives the requests.

- A vectored interrupt controller identifies the source of the interrupt.

- The CPU directly jumps to the corresponding Interrupt Service Routine (ISR).

- The ISR reads the sensor status/data register.

- Sensor data is stored in memory using efficient buffering.

- The ISR is kept short to reduce CPU utilization.

- After processing, the CPU returns to the interrupted task.

Why Memory-Mapped I/O?

| Feature | Memory-Mapped I/O | I/O-Mapped I/O |
| --- | --- | --- |
| Register access | Load/store instructions | Special I/O instructions |
| Address space | Shares memory address space | Separate I/O address space |
| Programming | Simple | More specialized |
| Latency | Low | Low, but architecture-dependent |
| Choice | Selected | Not selected |

Constraint Satisfaction

- CPU utilization: <15% by using short ISRs and buffering.

- Interrupt interval: 1 ms is supported.

- Interrupt handling: Vectored.

- I/O technique: Memory-mapped I/O.

Conclusion: A vectored interrupt controller combined with memory-mapped I/O and short ISRs provides a low-latency solution while keeping CPU utilization below 15%.

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

High-Speed I/O Subsystem Using NVMe and DMA

Problem

Design an I/O subsystem where:

- Storage throughput must be >3 GB/s

- NVMe over PCIe must be used.

- CPU should not handle bulk data transfer.

- DMA must be integrated with interrupt notification.

Proposed Architecture

NVMe SSD → PCIe → DMA Controller → Main Memory

CPU ← Interrupt Controller ← DMA/NVMe

Working

- The CPU sends an I/O command to the NVMe controller.

- NVMe communicates with the system through PCIe.

- The NVMe controller transfers data directly between the SSD and main memory.

- DMA handles the bulk data transfer.

- The CPU does not copy every data block.

- After completing the transfer, the controller generates an interrupt.

- The CPU receives the interrupt and processes the completion status.

Architecture

CPU

│

Command / Completion

│

▼

NVMe Controller

│

PCIe Bus

│

▼

DMA Engine

│

▼

Main Memory

Why NVMe over PCIe?

NVMe is designed for high-speed storage and uses PCIe to provide high bandwidth and low latency.

Why DMA?

Without DMA:

SSD → CPU → Memory

The CPU would be heavily involved in transferring data.

With DMA:

SSD → DMA → Memory

The CPU mainly handles commands and completion notifications.

Constraint Satisfaction

- Throughput: Designed for >3 GB/s.

- Interface: NVMe over PCIe.

- CPU bulk transfer: Avoided using DMA.

- Completion: Interrupt notification.

Conclusion: NVMe over PCIe combined with DMA provides a high-speed storage subsystem while minimizing CPU involvement.

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

Multi-Device Communication Architecture

Problem

Construct an architecture with:

- 6 USB peripherals

- 2 high-speed Thunderbolt devices

- Avoid interrupt starvation.

- Use hardware and software interrupts.

- Ensure fair bus arbitration.

Proposed Architecture

CPU

│

Interrupt Controller

/              \

Hardware Interrupts    Software Interrupts

│

┌──────┴──────┐

│             │

USB Controller  Thunderbolt Controller

│             │

USB Hub       Thunderbolt Hub

/ / / / / \       /       \

6 USB devices   TB Device 1  TB Device 2

Working

USB Devices

The six USB peripherals connect through a USB hub/controller.

The USB controller manages communication with the individual peripherals.

Thunderbolt Devices

The two high-speed devices use a separate Thunderbolt controller.

This prevents high-speed traffic from being unnecessarily blocked by ordinary USB traffic.

Interrupt Management

A priority-based interrupt controller is used.

However, lower-priority devices must not be permanently blocked.

A round-robin/fair scheduling mechanism can be used for devices with similar priority.

Hardware Interrupts

Used for events requiring immediate attention, such as:

- Device completion

- Error conditions

- High-priority hardware events

Software Interrupts

Used for:

- Deferred processing

- Background device tasks

- Operating-system service requests

Fair Bus Arbitration

A fair arbitration mechanism can use:

USB1 → USB2 → USB3 → USB4 → USB5 → USB6

↓

Thunderbolt devices

Requests are served fairly rather than allowing one device to continuously occupy the bus.

Avoiding Interrupt Starvation

- Use interrupt priorities.

- Apply round-robin scheduling among lower-priority devices.

- Limit the time spent inside ISRs.

- Defer lengthy processing to software interrupts/background tasks.

- Give high-speed devices appropriate priority without permanently blocking USB devices.

Constraint Satisfaction

| Constraint | Solution |
| --- | --- |
| 6 USB peripherals | USB controller/hub |
| 2 Thunderbolt devices | Thunderbolt controller |
| Interrupt starvation | Priority + fair scheduling |
| Hardware interrupts | Device events |
| Software interrupts | Deferred/background processing |
| Fair bus arbitration | Round-robin/fair arbitration |

Conclusion: Separating USB and Thunderbolt traffic while using prioritized interrupts and fair arbitration provides reliable multi-device communication.

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

Data Acquisition System

Problem

Design a data acquisition system with:

- Continuous data stream of 500 MB/s

- Minimal CPU intervention

- Compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers.

Proposed Architecture

Data Source

│

▼

Acquisition Device

│

Memory-Mapped I/O

│

▼

DMA

│

▼

Main Memory

│

▼

CPU

Working

- The acquisition device continuously receives data.

- Its control/status registers are accessed through memory-mapped I/O.

- DMA is configured with the destination memory address.

- Data is transferred directly from the device to memory.

- The CPU is notified after a block transfer is completed.

- The CPU processes the collected data rather than handling every byte.

DMA vs Interrupt-Driven I/O

| Feature | DMA | Interrupt-Driven I/O |
| --- | --- | --- |
| CPU involvement | Very low | High |
| Large data transfer | Excellent | Less efficient |
| 500 MB/s stream | Suitable | Not preferred |
| Interrupt frequency | Lower | Potentially very high |
| CPU overhead | Low | High |
| Selected | Yes | No |

Why DMA?

At 500 MB/s, generating and servicing interrupts for small portions of data would create significant CPU overhead.

DMA can transfer large blocks of data directly to memory.

Memory-Mapped I/O

Device registers are assigned memory addresses:

Device Register

↓

Memory Address

↓

CPU Load/Store

The CPU can use normal memory-access instructions to configure and monitor the acquisition device.

Constraint Satisfaction

- Data rate: 500 MB/s

- CPU intervention: Minimal

- Comparison: DMA is superior for continuous high-speed transfer.

- Device registers: Memory-mapped I/O

Conclusion: DMA with memory-mapped device registers is the preferred solution for a 500 MB/s continuous data acquisition system.

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

Nested Interrupt Handling Mechanism

Problem

Construct an interrupt mechanism with:

- Nested interrupts

- Maximum interrupt latency <10 µs

- Vectored interrupts for high-priority devices

- Software interrupts for background tasks.

Proposed Architecture

Devices

│

▼

Interrupt Controller

/            \

High Priority       Low Priority

│                    │

Vectored ISR       Normal ISR

│                    │

└────────┬───────────┘

▼

CPU

│

▼

Software Interrupts

Background Tasks

Working

- Devices generate interrupt requests.

- The interrupt controller determines their priority.

- A high-priority interrupt is immediately serviced.

- The CPU saves the state of the currently executing task.

- The high-priority ISR executes.

- If an even higher-priority interrupt arrives, nested interrupt handling allows it to interrupt the current ISR.

- After completion, the CPU restores the previous context.

- Lower-priority/background work is handled using software interrupts.

Vectored Interrupts

For high-priority devices:

Interrupt → Vector Number → ISR Address

The CPU can directly locate the appropriate ISR instead of searching for the interrupt source.

This reduces interrupt handling time.

Nested Interrupts

Example:

Normal Task

↓

Low Priority ISR

↓

High Priority Interrupt

↓

High Priority ISR

↓

Return to Low Priority ISR

↓

Return to Normal Task

Software Interrupts

Software interrupts are used for deferred or background tasks so that hardware ISRs remain short.

Achieving <10 µs Latency

The design should:

- Use vectored interrupts.

- Keep ISRs short.

- Use hardware priority levels.

- Save/restore context efficiently.

- Avoid unnecessary processing inside hardware ISRs.

- Defer non-critical work to software interrupts.

Constraint Satisfaction

| Constraint | Solution |
| --- | --- |
| Nested interrupts | Priority-based nested ISR |
| Latency <10 µs | Fast vectored interrupt handling |
| High-priority devices | Vectored interrupts |
| Background tasks | Software interrupts |

Conclusion: A priority-based nested interrupt system with vectored handling for critical devices and software interrupts for deferred work can satisfy the required low-latency interrupt architecture.

Code of Conduct:

I ____________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |