SIMATS ENGINEERING

ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA1202 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | P Dinesh Reddy | Reg.No.: | 192525399 |
| Department: | AIML | Date: |  |

Code of Conduct:

I P Dinesh Reddy 192525399 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

ANNEXURE A

COMPARATIVE ANALYSIS

1.Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

2.Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

3.Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

4.Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

5.Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

# Compare L1, L2, and L3 Cache Levels

Cache memory is a high-speed memory placed between the CPU and main memory. Modern processors generally use three cache levels: L1, L2, and L3. They differ mainly in size, latency, bandwidth, and their effect on processor performance.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Inside CPU core | Near/inside CPU core | Shared among cores |
| Size | Smallest | Medium | Largest |
| Access Latency | Lowest | Higher than L1 | Higher than L2 |
| Bandwidth | Highest | High | Lower than L1/L2 |
| Throughput | Very high | High | Moderate |
| Cost per bit | Highest | High | Lower |
| Typical Use L1 Cache: | Instructions and frequently used data | Backup for L1 | Shared data and backup for L2 |

L1 is the fastest and smallest cache. It is usually divided into L1 instruction cache (I-cache) and L1 data cache (D-cache). Because it is very close to the CPU execution units, its access latency is very low.

L2 Cache:

L2 is larger than L1 but slightly slower. If the required data is not found in L1, the processor searches L2. It provides a balance between capacity and speed.

L3 Cache:

L3 is generally the largest cache and is often shared between multiple processor cores. It has higher latency than L1 and L2 but reduces the number of accesses to slower main memory.

Impact on Processor Performance:

The cache hierarchy improves performance because frequently used instructions and data can be accessed without going to DRAM.

The average memory access time can be represented as:

AMAT = Hit Time + (Miss Rate × Miss Penalty) For multiple cache levels:

AMAT = T₁ + MR₁[T₂ + MR₂(T₃ + MR₃ × TM)] Where:

- T₁ = L1 access time

- MR₁ = L1 miss rate

- T₂ = L2 access time

- MR₂ = L2 local miss rate

- T₃ = L3 access time

- MR₃ = L3 local miss rate

- TM = Main memory access time

Thus, L1 provides the fastest access, L2 reduces L1 miss penalties, and L3 reduces expensive DRAM accesses. The hierarchy increases overall processor throughput and reduces CPU waiting time.

# Differentiate SRAM and DRAM

SRAM (Static RAM) and DRAM (Dynamic RAM) are important memory technologies.

SRAM is mainly used for cache memory, while DRAM is mainly used for main memory. The assessment specifically asks for comparison based on latency, bandwidth, and throughput.

| Feature | SRAM DRAM |  |
| --- | --- | --- |
| Full Form | Static Random Access Memory Dynamic Random Access Memory |  |
| Storage Cell | Flip-flop | Capacitor + transistor |
| Latency | Very low | Higher |
| Bandwidth | Very high | High |
| Throughput | Very high | High |
| Refresh | Not required | Required |
| Density | Low | High |
| Cost | Expensive | Cheaper |
| Main Application CPU Cache |  | Main Memory |

SRAM:

SRAM stores each bit using a flip-flop circuit. It does not require periodic refreshing as long as power is supplied.

Its major advantages are:

- Very low access latency

- High bandwidth

- High throughput

- Suitable for frequent CPU accesses

Therefore, SRAM is used in L1, L2, and L3 cache memories.

DRAM:

DRAM stores each bit using a capacitor. Since the capacitor gradually loses charge, DRAM requires periodic refreshing.

Advantages include:

- High storage density

- Lower cost per bit

- Large memory capacity

However, DRAM has higher latency than SRAM.

Why SRAM is Used for Cache:

The processor requires extremely fast access to frequently used instructions and data. SRAM provides the required low latency.

DRAM is used as main memory because a computer requires a much larger capacity than cache can practically provide.

Conclusion:

SRAM → faster, lower latency, expensive, smaller capacity → cache

DRAM → slower, higher capacity, cheaper → main memory

Thus, the combination of SRAM cache and DRAM main memory provides a good balance between speed, capacity, and cost.

# Compare Virtual Memory with Paging and Cache Memory

Virtual memory and cache memory both improve memory management and performance, but they work at different levels. The question specifically requires comparison in terms of access latency and system throughput.

| Feature Virtual Memory with Paging Cache Memory Main Purpose Provides large logical address space Speeds up memory access |  |  |
| --- | --- | --- |
| Managed By | OS + MMU | Hardware |
| Basic Unit | Page | Cache block/line |
| Storage | RAM + secondary storage | SRAM |
| Latency | High when page fault occurs | Very low on cache hit |
| Throughput | Lower during page faults | High |
| Speed | Slower | Much faster |
| Main Benefit | Memory capacity and isolation | Processor performance |

Virtual Memory with Paging:

Virtual memory allows a process to use a logical address space larger than the available physical RAM.

Memory is divided into fixed-size pages, while physical memory is divided into frames.

A virtual address contains:

Virtual Address = Page Number + Page Offset

The page number is translated into a physical frame number using the page table.

If the required page is not present in RAM, a page fault occurs. The operating system must load the page from secondary storage.

This greatly increases access latency.

Cache Memory:

Cache stores frequently or recently accessed data close to the processor.

If data is found: Cache Hit → Fast access If data is not found:

Cache Miss → Access next memory level

Cache performance can be represented using:

AMAT = Hit Time + Miss Rate × Miss Penalty A high cache hit rate results in high system throughput.

Access Latency Comparison:

Cache hit latency is extremely low because cache uses fast SRAM and is located close to the processor.

A page fault is much slower because data may need to be transferred from secondary storage.

Therefore:

Cache access << DRAM access << Page-fault handling Conclusion:

Cache memory primarily improves speed and CPU throughput, while virtual memory with paging primarily provides memory capacity, protection, and process isolation. Both are important but solve different problems.

# Compare NAND Flash and DRAM

NAND Flash and DRAM are semiconductor memory technologies with very different characteristics. The question asks for comparison based on latency, bandwidth, and suitability for hierarchical memory systems.

| Feature | NAND Flash DRAM |
| --- | --- |
| Type | Non-volatile Volatile |
| Latency | High Low |
| Bandwidth | High for sequential operations High |
| Throughput | Good for storage workloads Very high for memory workloads |
| Refresh | Not required Required |
| Persistence | Retains data without power Loses data without power |
| Cost per bit | Low Higher |
| Main Application SSD, storage Main memory |  |

NAND Flash:

NAND Flash is non-volatile, meaning it can retain stored information even when power is removed.

It is commonly used in:

- SSDs

- USB drives

- Memory cards

- Embedded storage

However, NAND Flash has much higher access latency than DRAM, especially for random access operations.

DRAM:

DRAM is volatile memory used as the main memory of computers.

It provides much faster random access compared with NAND Flash and is therefore suitable for active program execution.

Latency:

DRAM has much lower access latency than NAND Flash.

Therefore:

DRAM latency < NAND Flash latency

This makes DRAM suitable for CPU memory operations, while NAND Flash is better suited for persistent storage.

Bandwidth and Throughput:

DRAM provides high memory bandwidth for continuous CPU and application accesses.

Modern NAND-based SSDs can provide high sequential throughput, but their random-access latency is still significantly higher than DRAM.

Hierarchical Memory Suitability A typical hierarchy is:

CPU → L1 → L2 → L3 → DRAM → NAND Flash/SSD

DRAM is positioned close to the processor because it provides relatively low latency.

NAND Flash is placed lower in the hierarchy because it provides large, persistent, and economical storage.

Conclusion:

DRAM is suitable for main memory, while NAND Flash is suitable for secondary/persistent storage. They complement each other in a hierarchical memory system by balancing speed, capacity, persistence, and cost.

# Compare MRAM and RRAM

MRAM and RRAM are advanced non-volatile memory technologies. The assessment asks to compare them based on access time, throughput, and energy efficiency.

| Feature MRAM | RRAM |
| --- | --- |
| Full Form Magnetoresistive RAM | Resistive RAM |
| Storage Principle Magnetic state | Resistance state |
| Volatility Non-volatile | Non-volatile |
| Access Time Very fast | Fast |
| Throughput High | High |
| Energy Efficiency High | Very high potential |
| Endurance Very high | High, technology-dependent |
| Scalability Good | Very good potential |
| Embedded memory, Applications caches | Storage, embedded memory, neuromorphic systems |

MRAM:

MRAM stores information using magnetic states rather than electrical charge.

The major advantages of MRAM include:

- Non-volatility

- Fast read/write operation

- High endurance

- Low standby power

- Good energy efficiency

Since data remains stored without continuous power, MRAM can reduce energy consumption during standby.

RRAM:

RRAM stores information by changing the resistance of a material.

It uses different resistance states to represent binary information.

RRAM offers:

- Non-volatility

- High density

- Low-power operation

- Fast switching potential

- Good scalability

RRAM is also being investigated for neuromorphic computing and AI hardware because resistance states can represent more than simple binary information in some implementations.

Access Time:

Both technologies can provide fast access compared with conventional non-volatile storage technologies.

Lower access time → faster memory response

MRAM is particularly attractive where fast read/write operations and high endurance are required.

Throughput:

Throughput represents the amount of data that can be processed or transferred within a given time.

Both MRAM and RRAM have the potential for high throughput, but the actual throughput depends on:

- Memory architecture

- Device technology

- Read/write circuitry

- Parallelism

- Manufacturing process  Energy Efficiency:

Both technologies can reduce standby power because they are non-volatile.

MRAM provides good energy efficiency due to its non-volatile storage and fast operation.

RRAM has strong potential for extremely low-energy and high-density memory architectures.

Overall Comparison:

MRAM: Fast + high endurance + energy efficient

RRAM: Fast + high density + low-power potential + scalable

Therefore, MRAM is attractive for high-performance embedded memory, while RRAM is promising for high-density, low-power memory and emerging computing architectures.

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |